1,3-Dichloropropane — Data Compilation

CAS No.: 142-28-9​ | EINECS: 205-531-3​ | Molecular Formula: C₃H₆Cl₂​ | MW: 112.99

1) Basic Identification

| Item | Content |
| --- | --- |
| Chinese Name​ | 1,3-二氯丙烷 |
| Chinese Synonyms/Aliases​ | 氯化三亚甲基、二氯三亚甲基、Trimethylene dichloride、1,3-Propylene chloride |
| English Name​ | 1,3-Dichloropropane |
| SMILES​ | ClCCCCl |
| InChIKey​ | YHRUOJUYPBUZOS-UHFFFAOYSA-N |
| RTECS​ | TX9660000 |
| Customs Code (CN)​ | 29031990 |

2) Physicochemical Properties (Compiled from ChemicalBook page + ICSC + GESTIS cross-validation)

| Property | Value | Remarks/Source Discrepancies |
| --- | --- | --- |
| Appearance/Odor​ | Colorless transparent liquid; sweet/chloroform-like odor​ | Clear colorless liquid, sweet/chloroform-like |
| Melting Point​ | −99 °C​ (lit. −99.5 °C) |  |
| Boiling Point​ | 120–122 °C​ (lit. 120.4 °C) | Consistent across sources |
| Density (water=1)​ | ~1.19 g/mL (25 °C); at 20/4℃ approx. 1.1896​ | ρ ≈ 1.188–1.20 |
| Relative Vapor Density (air=1)​ | ~3.9​ | Vapor is much heavier than air |
| Vapor Pressure (20 °C)​ | ~2.4 kPa​ (≈18–24 hPa / 24.265 hPa @20–25 °C) | Unit conversions vary |
| Flash Point​ | 32 °C (closed cup, mostly reported as 90 °F≈32 °C); also reported open cup 16 °C​ | ⚠️ Means flammable vapors can form near room temperature; some sources report 20–22 °C |
| Autoignition Temperature​ | ~480 °C​ |  |
| Explosive Limits (in air)​ | LEL ~3.4% / UEL ~14.5%​ | Flammable liquid, vapor/air mixtures explosive |
| Refractive Index n²⁰/D​ | 1.448​ (lit. 1.4471–1.4491) |  |
| Water Solubility​ | Slightly soluble: ≈ 0.8 g/L (20 °C)​ = ~0.08 g/100mL (also noted as 0.27 g/100mL) | Essentially insoluble |
| Soluble in​ | Miscible with ethanol, ether, chloroform, ethyl acetate, and most organic solvents |  |
| LogP / Kow​ | log P ≈ 2.0​ (moderate hydrophobicity) |  |
| Dielectric Constant​ | ≈ 9.51​ |  |

Key Point: Flash point ~30–32 °C, so even if classified as "flammable liquid class 3" under some standards, during summer/warm lab operations it must be strictly handled as a flammable substance with rigorous fire source control + anti-static measures.

3) Safety Hazards & GHS (Summarized per mainstream SDS/ICSC for this substance)

Hazard Classification & Transport

| Item | Information |
| --- | --- |
| UN Number​ | UN 1993​ (Flammable liquids, n.o.s.) / Also UN 1279​ in specific manufacturer SDSs |
| DOT/ADR Class​ | 3 (Flammable liquids); PG II​ (few classify as PG III) |
| ICSC Number​ | 0724​ |
| WGK (German Water Hazard Class)​ | WGK 2​ (hazardous to water) |

GHS Pictograms & Hazard Statements (Mainstream Wording)

| GHS | Signal Word | Main H-Statements |
| --- | --- | --- |
| 🔥 GHS02 | Warning / Danger​ | H226​ – Flammable liquid and vapor |
| ⚠️ GHS07 |  | H315​ – Causes skin irritation; H319​ – Causes serious eye irritation; H335​ – May cause respiratory irritation |
| (Added in some SDSs) |  | Thermal decomposition → Toxic fumes including HCl, phosgene, etc. |

Carcinogenicity/Mutagenicity: Listed under "Mutagen" in RTECS; but IARC Group 3 (not classifiable), EPA generally does not list as a major carcinogen (unlike its "sibling" 1,3-dichloropropene 542-75-6) → Still should be handled according to specifications for harmful halocarbons, not to be taken lightly.

Toxicological Clues: Dog oral LDL₀ ≈ 3000 mg/kg; CNS depressant/anesthetic effects; irritates eyes, skin, respiratory tract; repeated contact may affect liver/kidneys.

4) First Aid / Firefighting / Spill Response / Storage (Per MSDS info on the page & ICSC)

First Aid

| Route | Treatment |
| --- | --- |
| Inhalation​ | Move to fresh air immediately; if breathing is difficult, administer O₂; if no breathing → CPR; seek medical attention​ |
| Skin​ | Remove contaminated clothing → rinse skin with plenty of running water and soap for ≥15 min → seek medical advice |
| Eyes​ | Immediately flush eyes with plenty of water or saline for ≥10–15 min (remove contact lenses if present); seek prompt ophthalmologic care​ |
| Ingestion​ | If conscious: rinse mouth; give small amount of water or milk; do NOT induce vomiting; call poison center/seek medical help |

Firefighting

Extinguishing Media: Alcohol-resistant foam / CO₂ / dry chemical / water fog (direct water stream may cause splashing/spread)

Hazardous Combustion Products: HCl, CO/CO₂, possible phosgene​ → Firefighters need SCBA + full protective clothing

Containers may rupture from heat → Cool with water spray but keep distance

Spill Response

1.Eliminate ignition sources / static / heat sources; evacuate personnel upwind;

2.Responders wear organic vapor respirator + chemical protective suit;

3.For small spills: absorb with sand/inert absorbent, collect into sealed waste container; prevent entry into sewers/water bodies;

4.For large spills: follow UN 1993​ flammable liquid protocols for containment/recovery.

Storage & Incompatibilities

Store in cool, ventilated, fire-proof area; tightly sealed; storage temp recommended ≤30 °C

Keep away from: Oxidizers, strong acids, strong bases, active metals like aluminum/magnesium​ (may increase corrosion/reaction risk)

Ground for static electricity; use non-sparking tools; provide secondary containment

5) Uses (As described on ChemicalBook page)

Organic synthesis intermediate (for synthesizing 1,3-propanediol derivatives, heterocycles/cyclopropane systems, etc.)

Solvent / detergent component

Matrix for Certified Reference Material (CRM) / standard solutions​ in Gas Chromatography (GC) analysis